Lab No:7

Experiment Name: DHCP Configuration in Cisco Router using Packet Tracer

Question:
Configure a DHCP server on a Cisco router to automatically assign IP addresses to multiple PCs in a network. Explain how DHCP simplifies IP address management compared to manual configuration.

Apparatus Required:

- Personal Computer/Laptop (to run simulation)

- Cisco Packet Tracer software

- Router (2 unit – for DHCP server)

- Switch (1 unit – to connect PCs)

- 4–6 PCs (depending on network size)

- Ethernet cables (straight-through)

Tools Used:

- Cisco Packet Tracer (for network simulation and DHCP configuration)

### Theory

DHCP (Dynamic Host Configuration Protocol):
DHCP is a network protocol used to automatically assign IP addresses and other network configuration parameters (such as subnet mask, default gateway, and DNS server) to devices in a network. It eliminates the need for manual IP configuration, reducing human error and simplifying network management. A router can act as a DHCP server, dynamically allocating IP addresses to connected devices from a predefined pool.

When a device (client) connects to the network, it sends a DHCP Discover message. The DHCP server responds with a DHCP Offer, the client sends a DHCP Request, and the server finalizes the process with a DHCP Acknowledgment (ACK). This is known as the DORA process (Discover → Offer → Request → Acknowledge).

ip dhcp pool [name]

- Purpose:

  - This command is used to create a DHCP pool on the router.

  - A DHCP pool is a range of IP addresses that the router can assign dynamically to clients (PCs, laptops, etc.) on the network.

- Syntax:

ip dhcp pool <pool-name>

Additional Configurations Inside the Pool:

- Network/Subnet: Specifies the IP range and subnet mask.

Eg:network 192.168.3.0 255.255.255.0

- Default Gateway: Sets the router IP for clients as their default gateway.

Eg: default-router 192.168.3.1

Lab diagram:

Procedure

- Set up the Physical Topology:

  - Open Cisco Packet Tracer.

  - Place two Router on the workspace (they will act as the DHCP server).

  - For each router, place one Switch to connect all the end devices.

  - Add 4 pcs as end devices to each switch.

  - Connect the devices using straight-through Ethernet cables as follows:

    - Router → Switch (Fast Ethernet interface)

    - Each PC → Switch (Fast Ethernet interface)

  - Ensure all cables are correctly connected

2.Router Configuration:

On router 1(Prajwal-rtr):

In router2(Prajwal-rtr1):

3. Configuring DHCP on the Router

In router1(Prajwal-rtr):

In router2(Prajwal-rtr1):

4. Configure PCs for Dynamic IP Addressing

In network1(192.168.2.0):

Testing:

1. Test Connectivity (Ping Test) from pcs to their respective routers:

for router1(Prajwal-rtr) in network1(192.168.2.0):

for router2(Prajwal-rtr1) in network2(192.168.3.0):

2. Check Router DHCP Binding:

In router1(Prajwal-rtr):

In router2(Prajwal-rtr1):

Conclusion

In this experiment, we successfully configured DHCP (Dynamic Host Configuration Protocol) on a Cisco router using Packet Tracer. By setting the PCs to obtain IP addresses dynamically, the DHCP server automatically assigned IP addresses, subnet masks, and default gateways from the predefined pool. This eliminated the need for manual IP configuration, reduced the chances of IP conflicts, and made network management easier and more efficient.